Emperical Fluid Dynamics & Dimensional Analysis

(DA)

Chapter 5 of F. White’s Fluid Mech Book

Learning Goals:

- Introduction to empirical (experimental Fluids Dyn.)

- Let’s look at two experiments:

- Drop impact onto a surface

- Drop transfer between to surfaces

- How to construct Dimensionless numbers

- How to use Dimensionless numbers in:

- Planning for experiments

- Interpreting the data from experiments

Similarity of Flow in Fluids Dynamics and its application

Why Study Drop Impact on Surfaces

Lubrication of big ship engines

A drop impacting a dry solid surface under normal atmospheric conditions!

T=0

T = 0.3 ms

T= 2.6 ms

T = 6.6 ms

T= 14.6 ms

T =750 ms

A drop on to solid dry isothermal surfaces

# Influence of Gas Pressure

# Surface roughness influence

(silicon oil)

# Wettability influence on drop impact

# Isothermal Impacts of Newtonian drops

# Drop Impact parameters: how to reduce them

U,D, 𝜌,𝜎,𝜇,𝑔,𝑃𝑎𝑚𝑏, h , 𝑅𝑎, 𝜆𝑅, 𝜃, 𝐸and time t

- We (inertial/superficial forces) 𝑊𝑒 = 𝜌𝑉2𝐷𝜎

- Oh (viscous / superficial forces) 𝑂h = 𝜇𝜌𝜎𝐷

- Re (inertial viscous forces) 𝑅𝑒 = 𝜌𝑉𝐷𝜇= 𝑊𝑒𝑂h

Example: atmospheric Water Drop

- V = 50 M/S

- D = 𝜇 m

- We = 1700

- Oh = 0.03

# DA for Data Analysis/Interpretation

Example of liquid bridge case for printing industry:

# Industrial Application

Other applications

- Dispensing of glue for packaging

- Electro wetting-assisted drop deposition

- Micromachined fountain-pen techniques

In Nature:

Cat laps

Liquid adhering to the dorsal side of the tongue tip is drawn upward, forming a liquid bridge.

Process of liquid transfer from one surface to another

Typical process of liquid transfer

Transfer ratio-

𝛼=𝑉1𝑉1+𝑉2

Governing parameters for the transfer ratio:

- Donor Surface: Teflon AF

- Acceptor surface: PMMA

- Stretching speed (U): 25 mm/s

- Donor surface: Teflon AF

- Acceptor surface: PMMA

- Stretching speed (U): 1mm/s

- Donor surface : Silicon

- Acceptor Surface: PMMA

- Stretching speed: 1 mm/s

# What maybe parameters of interest?

- liquid viscosity

- Separation velocity of surfaces

- Surface tension of liquid

- Type of surfaces

20 cSt silicone oil:

100 cSt silicone oil:

# Roles of liquid viscosity

Four types of silicon oil transfer from Teflon AF to PEMA

What nondimensional parameters we can use?

Capillary number Ca (Ca = 𝜇𝑉𝜎) [viscos forces over surface tension forces]

Transfer Ratio as Function of Lof (Ca)

Effects of liquid surface tension

Goals for today’s lecture – Lecture 10

- Introduction of 3rd way to study fluid dynamics, i.e., empirical studies

- Discussion of an experiment and parameters that can be involved

- How to handle many parameters tjat can be involved in an empirical study

- Dimensional Analysis as a way of interpreting data and managing experiment parameters in a study

Chapter 5: Dimensional Analysis (DA) & Similarity

DA id basically a way to reduce the numbers of variables to allow for planning on data interpretation in/from an experimental worK.

DA saves time and money, as we can reduce the number of experimental runs. Experimental work is an important part of F.D. as many cases are too complex in mission critical to use analytical or CFD methods (alone).

Note: Read Sec. 5.1 & 5.2 \

Sec. 5.3 The Pi Theorem

We will use Ipsen method rather thanBuckingham method to do the DA. In DA we create non dimensional compound variables, out of dimensional variables. The non-dimensional compund varibales are called Pi (due to symbol - π).

If phenomenon (e.g. drag on airfoil) depends on “n” dimensional variables (e.g. velocity, density,etc), the DA reduces the problem to “K” dimenisonaless parameters (e.g. C_D & R_e).

Generally n – k is equal to number of fundamental (or basic) dimensions (e.g. M,L,T, etc) something

Do you know some non-dimensional variables?

𝑅𝑒 = 𝜌𝑉𝐿𝜇𝐿 ⇒𝑐h𝑎𝑟𝑎𝑐𝑡𝑒𝑟𝑖𝑠𝑡𝑖𝑐 𝑙𝑒𝑛𝑔𝑡h 𝑓𝑜𝑟 𝑡h𝑒 𝑠𝑦𝑠𝑡𝑒𝑚, 𝑀𝑎 = 𝑉𝑎, 𝐾 = 𝐶𝑝𝑐𝑣 (specific heat)

The non-dimensional varibales usually have or signify physical meaning, e.g.

Re =  𝑖𝑛𝑒𝑟𝑡𝑖𝑎𝑙 𝑓𝑜𝑟𝑐𝑒𝑠𝑣𝑖𝑠𝑐𝑜𝑢𝑠 𝑓𝑜𝑟𝑐𝑒𝑠⇒signify how much inertial forces should be longer than viscours forces to transmit turbulent flow.

𝑊𝑒 = 𝑠𝑣2𝐿𝑟 = 𝑖𝑛𝑒𝑟𝑡𝑖𝑎𝑙 𝑓𝑜𝑟𝑐𝑒𝑠𝑠𝑢𝑟𝑓𝑎𝑐𝑒 𝑡𝑒𝑛𝑠𝑖𝑜𝑛 𝑓𝑜𝑟𝑐𝑒𝑠 (We #)

𝐵0 = 𝜌𝑔𝐿2𝑟 = 𝑔𝑟𝑎𝑣𝑖𝑡𝑦 𝑓𝑜𝑟𝑐𝑒𝑠𝑠𝑢𝑟𝑓𝑎𝑐𝑒 𝑡𝑒𝑛𝑠𝑖𝑜𝑛 𝑓𝑜𝑟𝑐𝑒𝑠, 𝑘  = 𝑐𝑝𝑐𝑣 = 𝑒𝑛𝑡h𝑎𝑙𝑝𝑦𝑖𝑛𝑡𝑒𝑟𝑛𝑎𝑙 𝑒𝑛𝑒𝑟𝑔𝑦

Drag coefficient 𝐶𝐷 = 𝐷12𝜌𝑣2𝐿 = 𝑑𝑟𝑎𝑔 𝑓𝑜𝑟𝑐𝑒𝑖𝑛𝑒𝑟𝑡𝑖𝑎 𝑓𝑜𝑟𝑐𝑒

- How to use this information to make plans for experiments?

- How to use this information to interpreat data?

- Planning: should I worry about liquid surface tension when designing a channel flow?

Channel dimensions: 30 x 20 xm and 2m long

Flow velocity: 0.1 m/s

Liquid: water or ethanol 𝜌𝑤 = 1000 𝑘𝑔𝑚3 , 𝜌𝑒 = 790 𝑘𝑔𝑚3 , 𝑟𝑤 = 72 𝑚𝑁𝑚, 𝑟𝑒 = 22𝑚𝑀𝑚

Which # to use? We

𝑊𝑒𝑤 = 1000 𝑥 10−2 𝑥 3 𝑥 10−172 𝑥 10−3 = 41.7 ⇒Inertia is N42 times more important than surface forces.

What if the channel is angled (effect of gravity) ?

𝐵0𝑤 = 1000 𝑥 9.8 𝑥 3𝑥10−1272 𝑥 10−5  = 12250⇒So not important at all

𝐵𝑜𝑒 = 31,672

How about viscous forces?

𝑅𝑒𝑤 = 𝜌𝑈𝐿𝜇 = 200,000⇒Not to worry about viscous forces ,

What if the velocity B is reduced to 1mm/min -> 𝑅𝑒𝑤 ≈23, 𝑅𝑒𝑒 ≈26 → now vicous forces are more important.

How about surface forces ?

𝑊𝑒𝑤≈2.3 𝑥 10−4→Surface forces are the King/Queen in µ channel.

So such planning analysis will allow us to plan betetr; for a large channel surface tension will not be a design/experiment consideration, but for a micro-channel it should be!

- Once data is collected in an experiment, say rag force coefficient for a sphere or a cycle. Non-dimensional plot, allows a comprehensive representation of data; as seen in figure below, the plots are independent of the fluid used (e.g.,µ,𝜌 ), so it can be for air, water, oil, etc. No more rad for 3 plots for air, oil, water...

Also, data interpretation in terms of e.g. forces, with a plot such as the one above can be made in general fashion (e.g. regardless of the type fo fluid); see encircled areas in the above plot where an increase inertoal forces leads to reduction of forces on the cycle or spherical body in any fluid flow (𝐶𝐷↓)

How to make non-dimensional groups (π) or numbers.